AI303L: Computer Vision Lab

Assignment 1 Report

MRI Filtering and Image Quality Check

Submitted By

| Muhammad Maaz Akram | 231192 |
| --- | --- |
| Muhammad Umar | 231200 |

BSAI-6-A

Submitted To

Ms. Hina Rashid

Prepared on 18 May 2026

# Introduction

In this assignment, we worked on MRI brain images and applied basic image filtering techniques. The main purpose was to see how different filters change the image and how much detail they preserve. This is an important step in computer vision because medical images usually need some preprocessing before further tasks like segmentation or classification.

Three filters were tested: Gaussian filter, mean filter, and median filter. After applying each filter, the results were compared with the original normalized image using PSNR and SSIM. The output images were also saved so that the results could be checked visually.

# Dataset Used

The main dataset selected for this work was the BraTS-PEDs dataset from The Cancer Imaging Archive. The full dataset is very large, so it was not practical to download and process the complete collection again and again during the lab work. To make the work manageable, a smaller chunk of the dataset was uploaded to Kaggle and then used inside the code through the KaggleHub API.

So, the original source of the data is TCIA BraTS-PEDs, but the actual files used by the notebook came from the Kaggle dataset named awansaad6797/cancer-dataset. This is why the code contains the KaggleHub download command. It allowed the notebook to work with a smaller and easier-to-handle part of the original dataset.

| Dataset Point | Explanation |
| --- | --- |
| Original dataset | BraTS-PEDs from The Cancer Imaging Archive |
| Original dataset link | https://www.cancerimagingarchive.net/collection/brats-peds/ |
| Dataset DOI | 10.7937/dx5c-tj86 |
| Full dataset size | About 457 subjects and around 32.7 GB of image data |
| Dataset used in code | Kaggle chunk accessed with KaggleHub |
| Kaggle dataset name | awansaad6797/cancer-dataset |
| Reason for Kaggle use | The full dataset was large, so a smaller uploaded chunk was used for the assignment. |

# Objectives

Load MRI volumes from the Kaggle dataset chunk.

Select useful center slices from each volume.

Normalize the image intensity values.

Apply Gaussian, mean, and median filters.

Save original, filtered, and comparison images.

Use PSNR and SSIM to compare the filters.

# Methods

The notebook first downloaded the dataset using KaggleHub. Then it searched for MRI files and loaded them with NiBabel. If a volume had four dimensions, the first channel was used. For every volume, center slices were selected because they usually contain more useful brain information than slices near the edges.

Each slice was normalized between 0 and 1. This made the image values consistent before filtering. After that, three filters were applied. The Gaussian filter used sigma 1.0, while the mean and median filters used a 3 by 3 window.

| Filter | What It Does | Parameter Used |
| --- | --- | --- |
| Gaussian | Smooths the image using weighted nearby pixels. | Sigma = 1.0 |
| Mean | Replaces each pixel with the average of nearby pixels. | 3 by 3 window |
| Median | Replaces each pixel with the middle value of nearby pixels. | 3 by 3 window |

# Evaluation Metrics

PSNR was used to measure how close the filtered image was to the original image. A higher PSNR means less difference from the original image. SSIM was used to check how much structure was preserved. A higher SSIM means the important image structure is more similar to the original.

# Results

| Item | Details |
| --- | --- |
| Notebook used | CV_Project_Milestone_1.ipynb |
| Total metric rows | 4,800 |
| Unique labels in metrics | 13 |
| Saved comparison images | 56 |

| Filter | Rows | Mean PSNR | PSNR Range | Mean SSIM | SSIM Range |
| --- | --- | --- | --- | --- | --- |
| Gaussian | 1,600 | 31.5317 | 26.7367 to 37.4594 | 0.9484 | 0.8720 to 0.9793 |
| Mean | 1,600 | 32.1203 | 27.0110 to 38.4006 | 0.9515 | 0.8614 to 0.9846 |
| Median | 1,600 | 34.6233 | 28.8928 to 41.1842 | 0.9639 | 0.8712 to 0.9922 |

Figure 1. Average PSNR for the three filters.

Figure 2. Average SSIM for the three filters.

## Processed Labels

| Label | Rows | Average PSNR | Average SSIM |
| --- | --- | --- | --- |
| T1CE_to_SRI_defaced | 1140 | 33.589 | 0.9591 |
| FL_to_SRI_defaced | 1140 | 31.559 | 0.9393 |
| T1_to_SRI_defaced | 1020 | 33.290 | 0.9691 |
| T2_to_SRI_defaced | 960 | 32.172 | 0.9469 |
| BraTS-PED-00195-000-t1n | 60 | 35.083 | 0.9823 |
| BraTS-PED-00195-000-t2w | 60 | 38.356 | 0.9777 |
| sub002_t1ce_defaced | 60 | 33.314 | 0.9516 |
| sub002_t1_defaced | 60 | 32.843 | 0.9686 |
| sub002_fl_defaced | 60 | 32.339 | 0.9604 |
| sub002_t2_defaced | 60 | 32.589 | 0.9623 |
| BraTS-PED-00201-000-t1n | 60 | 33.660 | 0.9782 |
| BraTS-PED-00201-000-t2w | 60 | 32.993 | 0.9593 |
| BraTS-PED-00203-000-t2w | 60 | 30.996 | 0.9342 |

# Visual Results

The following images are taken directly from the output folder. Each figure shows the original slice and the filtered versions. These figures make it easier to see the difference between the filters instead of only looking at numbers.

Figure 3. T1CE slice comparison.

Figure 4. T1 slice comparison.

Figure 5. T2 slice comparison.

Figure 6. FLAIR slice comparison.

Figure 7. BraTS-PED T2-weighted slice comparison.

# Discussion

From the results, the median filter performed the best overall. It gave the highest average PSNR and SSIM values. This means it changed the image less and preserved the structure better than the other two filters. This also makes sense visually, because median filtering removes small noise while keeping edges sharper.

The mean filter also smoothed the images, but it blurred some details because it simply averages nearby pixels. The Gaussian filter gave smoother results, but in this experiment it had lower average PSNR and SSIM than the median filter.

It is important to note that PSNR and SSIM compare the filtered image with the original image. So, a higher value means the image stayed closer to the original. It does not automatically mean that the filtered image is always better for medical diagnosis.

# Conclusion

This assignment showed how basic filters affect MRI images. The notebook successfully loaded the dataset chunk, selected slices, normalized them, applied three filters, saved the outputs, and calculated PSNR and SSIM. Based on both the numbers and the visual results, the median filter was the best option in this experiment.

# References

BraTS-PEDs dataset from The Cancer Imaging Archive: https://www.cancerimagingarchive.net/collection/brats-peds/

Kaggle dataset used in code: awansaad6797/cancer-dataset

SciPy ndimage documentation for image filters.

scikit-image documentation for PSNR and SSIM.